EE588 Homework 3: Notebook Results Summary for a 4-Channel C-Band AWG

Compiled from waveguide_mode.ipynb, waveguide_width_sweep.ipynb, and awg_initial_design.ipynb

Date: 2026-05-22

1. Report Objective

This document consolidates the main results from the project notebooks into a homework-style report. The goal is to show how the baseline silicon strip waveguide MODE study was extended into a width sweep and then translated into a first-pass 4-channel C-band AWG design.

2. Notebook Scope and Workflow

•	
      waveguide_mode.ipynb established a baseline MODE solution for a 500 nm x 220 nm silicon strip waveguide in SiO2.

•	
      waveguide_width_sweep.ipynb scanned widths from 0.40 um to 0.60 um at 1550 nm to extract n_eff and n_group.

•	
      awg_initial_design.ipynb converted the width-sweep result into channel targets, FSR, diffraction order, and array-length estimates for a simplified AWG.

3. Baseline MODE Result

The baseline notebook confirms that the selected silicon strip waveguide supports a fundamental guided mode near 1550 nm. The 0.50 um width case is used as the reference design point because it is already represented in the initial MODE run and sits near the middle of the width sweep range.

Item

Value

Core material

Silicon (n = 3.48)

Cladding material

SiO2 (n = 1.44)

Waveguide width

0.50 um

Waveguide height

0.22 um

Reference wavelength

1.55 um

Fundamental mode n_eff

2.4446

Fundamental mode n_group

4.0512

4. Width Sweep Results

A low-cost MODE sweep was run at 1550 nm for widths from 0.40 um to 0.60 um. The results show a monotonic increase in effective index and a gradual decrease in group index as the waveguide becomes wider.

Width (um)

n_eff

n_group

0.4

2.2275

4.2449

0.45

2.3527

4.1444

0.5

2.4446

4.0512

0.55

2.5131

3.9749

0.6

2.5657

3.9142

Based on these trends, 0.50 um is a practical first-pass choice. It matches the earlier baseline result, avoids the extremes of the scan range, and provides n_eff = 2.4446 and n_group = 4.0512 for subsequent AWG calculations.

5. AWG Initial Design Results

The AWG design notebook maps the single-waveguide results into a simplified 4-channel C-band demultiplexer target. The design uses a 1550 nm center wavelength and 1.6 nm channel spacing, which is approximately 200 GHz near 1550 nm.

Parameter

Value

Unit

channel_count

4

count

center_wavelength

1550

nm

channel_spacing

1.6

nm

target_fsr

19.2

nm

awg_order_m

81

dimensionless

actual_fsr

19.1358

nm

delta_length

30.9907

um

arrayed_waveguide_count

16

count

suggested_fpr_length

50

um

suggested_array_pitch

1.5

um

suggested_output_pitch

2

um

minimum_bend_radius

10

um

Channel

Center Wavelength (nm)

Frequency (THz)

Notes

1

1547.6

193.7144

Lower-wavelength edge channel

2

1549.2

193.5144

Inner channel

3

1550.8

193.3147

Inner channel

4

1552.4

193.1155

Higher-wavelength edge channel

The calculated path-length ladder grows in nearly uniform steps of about 30.99 um, reaching a relative difference of 464.86 um by the final arm. This makes the geometry easy to interpret and suitable for a first-pass AWG layout.

6. Related Figure Files

The notebook-produced figures are preserved alongside this report. They are listed here because the macOS textutil conversion path used for this DOCX did not embed local images reliably.

Figure File

Location Note

neff_vs_width.png

Generated by the notebook and kept in project/tidy3d_awg_project/report_assets/

simplified_awg_geometry_layout.png

Generated by the notebook and kept in project/tidy3d_awg_project/report_assets/

7. Conclusions and Next Steps

•	
      The notebook sequence provides a clean path from single-waveguide MODE analysis to AWG parameter estimation.

•	
      The 0.50 um waveguide width remains the recommended baseline because it balances continuity, moderate index values, and reportability.

•	
      The first-pass AWG target is a 4-channel C-band demultiplexer with channel centers at 1547.6, 1549.2, 1550.8, and 1552.4 nm.

•	
      The next project step should be to validate the simplified geometry and then decide whether to continue with low-cost approximations or move into device-level simulation.