EE588 Final Project Report

C-band 4-Channel AWG / Wavelength Demultiplexer Design Using Tidy3D

Date: 2026-05-26

1. Objective

This report consolidates the full three-week project workflow, from silicon strip waveguide MODE analysis to a first-pass 4-channel AWG design and a final low-cost spectral evaluation.

2. Workflow Summary

•	
      Week 1 established the baseline 500 nm x 220 nm silicon strip waveguide MODE solution and verified the fundamental TE-like guided mode.

•	
      Week 1 also completed a width sweep from 0.40 um to 0.60 um, producing the effective-index and group-index data used for AWG design.

•	
      Week 2 translated the MODE data into a 4-channel C-band AWG parameter set, including channel plan, FSR, diffraction order, and array path-length ladder.

•	
      Week 3 completed a simplified local spectrum analysis using the derived AWG parameters, yielding transmission, insertion-loss, crosstalk, and channel-spacing estimates.

3. Baseline Waveguide Result

Item

Value

Waveguide width

0.50 um

Waveguide height

0.22 um

Reference wavelength

1550 nm

Fundamental mode n_eff

2.4446

Fundamental mode n_group

4.0512

Width sweep result used to select the 0.50 um baseline.

4. First-Pass AWG Design

Parameter

Value

Unit

Channel count

4

count

Center wavelength

1550

nm

Channel spacing

1.6

nm

Target FSR

19.2

nm

Actual FSR

19.1358

nm

AWG order

81

dimensionless

Path length difference

30.9907

um

Arrayed waveguides

16

count

Channel

Center Wavelength (nm)

Center Frequency (THz)

1

1547.6

193.7144

2

1549.2

193.5144

3

1550.8

193.3147

4

1552.4

193.1155

First-pass simplified AWG geometry preview.

5. Final Simplified Spectral Evaluation

Because full device-level FDTD validation would consume substantially more Tidy3D credits, the final project stage uses a local array-factor approximation driven by the already extracted MODE parameters and AWG geometry design values.

This means the final spectra below should be interpreted as a system-level first-pass estimate rather than a claim of fully converged electromagnetic device performance.

Simplified transmission spectra for the four output channels.

Insertion loss, worst-case crosstalk, and 3 dB bandwidth by channel.

Channel

Target (nm)

Peak (nm)

Insertion Loss (dB)

Worst Crosstalk (dB)

3 dB BW (nm)

1

1547.6

1547.6

3

-13.71

1.06

2

1549.2

1549.2

3

-13.71

1.06

3

1550.8

1550.8

3

-13.71

1.06

4

1552.4

1552.4

3

-13.71

1.06

6. Key Final Results

•	
      The average adjacent channel spacing is 1.6 nm, which matches the 1.6 nm design target.

•	
      The average insertion loss in the simplified model is 3 dB.

•	
      The worst-case crosstalk in the simplified model is -13.71 dB.

•	
      The average 3 dB channel bandwidth is 1.06 nm.

•	
      All four channel peaks remain aligned with the intended center wavelengths in the simplified analysis output.

7. Conclusion

The project successfully progressed from low-cost single-waveguide MODE analysis to a reportable 4-channel C-band AWG design flow. The resulting parameter set is internally consistent, produces the intended channel spacing, and supports a coherent final submission package.

The main remaining improvement path is clear: a future iteration should replace the local spectral approximation with one or more device-level Tidy3D simulations to refine insertion loss, passband shape, and crosstalk with higher physical fidelity.